- SCOPE OF SUPPLY AND TECHNICAL SPECIFICATION

# Scope of Supply

The scope of supply will include but is not limited to the following:

  - Packing requirements

  - Document requirements

  - Compliance and Quality standards

| # | Preferred Product /Supplier | Chemical | Technical Specification | Packaging | Qty |
| --- | --- | --- | --- | --- | --- |
| 1 | MemAqua, PE Chemicals, Nuvest Chemicals, Protea Chemicals | Sodium Metabisulphite (SMBS) | Commercial Grade, Purity ≥99%, COA required | 25 kg bags on pallet | 20 bags |
| 2 | Water Component, Marsi, AR Africa | Antiscalant | Avista Vitec 7000 or approved equivalent | 1000 L IBC | 1 |
| 3 | Idwala, MemAqua, Tychem Chemicals | Hydrated Lime | Commercial Grade, COA required | Bulk tanker (26 ton) | 1 |
| 4 | Pulpchem, Buckman, Nuvest Chemicals | Bulab 5954 Flocculant | Buckman Bulab 5954 or approved equivalent | 25 kg bags | 20 bags |
| 5 | MemAqua, Nuvest Chemicals, Protea Chemicals | Sulphuric Acid | H₂SO₄ 71%, Commercial Grade | 1000 L IBC | 1 |
| 6 | Marsi, Pulpchem, Aquatrade Water Chemicals | Filter Aid | Avista RoQuest 3000 or approved equivalent | 1000 L IBC | 1 |

| 7 | MemAqua, Proxa | Biocide | MemClean20 (DBNPA 20%) or approved equivalent | 25 L Polycan | 20 |
| --- | --- | --- | --- | --- | --- |
| 8 | MemAqua Nuvest Chemicals, Protea Chemicals, PE Chemicals | Sodium Hydroxide | NaOH 45%, Commercial Grade | 25 L Polycan | 20 |
| 9 | MemAqua, Nuvest Chemicals, Protea Chemicals, PE Chemicals | EDTA Na₄ | Tetrasodium EDTA | 25 kg bags | 20 bags |
| 10 | Proxa, Protea Chemicals, PE Chemicals | Prochem Solaway | Proprietary Cleaning Chemical | 25 kg bags | 20 bags |

# Packing requirements

All packing shall comply with the following requirements in general

| Requirement | Specification |
| --- | --- |
| Packaging Condition | All packaging shall be new, undamaged and fit for transport and storage. |
| Product Protection | Packaging shall prevent contamination, moisture ingress, UV degradation and mechanical damage during transport and storage. |
| Transport Compliance | Packaging shall comply with applicable SANS standards, ADR/UN transport requirements (where applicable) and South African legislation governing hazardous chemical transport. |
| Rejection Criteria | Damaged, leaking, contaminated or incorrectly labelled packaging will be rejected at the supplier's cost. |

All packing shall comply with the following packing types

| Packaging Type | Specification |
| --- | --- |
| 25 kg Bags on Wooden Pallets | New or Grade A reusable hardwood/heat-treated timber pallets complying with ISPM 15. Nominal pallet dimensions 1200 mm × 1000 mm with a minimum Safe Working Load (SWL) of 1,200 kg. Four-way forklift entry suitable for pallet jack handling. Products to be supplied in sealed 25 kg moisture-resistant bags, uniformly stacked, stretch-wrapped with UV-resistant polyethylene film and fitted with corner protectors where required. Maximum pallet height 1.4 m. Each pallet shall be clearly labelled with product name, batch number, manufacturing date, net mass and applicable hazard warnings. |
| 1000 L Intermediate Bulk Container (IBC) | Nominal capacity 1,000 litres. Approximate dimensions 1200 mm (L) × 1000 mm (W) × 1160 mm (H). UV-stabilised HDPE inner bottle enclosed in a galvanised steel cage with galvanised steel or composite pallet base. UN-approved for transport of hazardous chemicals where applicable. Fitted with a 150 mm vented screw cap and 2-inch (DN50) |

Compliance and Quality requirements

Chemicals provided should be subject to the following requirements.

|  | bottom discharge valve complete with tamper-evident seal. Suitable for forklift handling and stackable (minimum two high when full). Each IBC shall display product identification, UN number, GHS hazard symbols, batch number, manufacturing date, expiry date (where applicable), net mass and supplier details. |
| --- | --- |
| 25 L Polycan | High Molecular Weight High Density Polyethylene (HM-HDPE) chemical-resistant container with nominal capacity of 25 litres. Approximate dimensions: 420 mm (H) × 265 mm (L) × 250 mm (W). Fitted with heavy-duty tamper-evident 60 mm screw cap and integral carry handle. Containers shall be UN-approved for hazardous chemicals where applicable and capable of passing UN drop, leak, hydraulic pressure and stacking tests. Containers shall withstand outdoor storage and normal handling without deformation or leakage. Each container shall be labelled with product name, concentration, batch number, manufacturing date, expiry date, UN number (where applicable) and GHS hazard labels. |
| Bulk Pneumatic Tanker (Hydrated Lime) | Hydrated lime shall be supplied in enclosed pneumatic bulk tankers with a nominal payload of 26 metric tonnes. Tankers shall be clean, dry and dedicated for lime transport to prevent contamination. Product shall be protected against moisture ingress during transport and unloading. Delivery shall be compatible with the existing pneumatic unloading facilities at the Nkomati Water Treatment Plant. Each delivery shall be accompanied by a weighbridge ticket, delivery note and Certificate of Analysis (COA). |

# Document requirements

Documentation Required with each delivery.

| Requirement | Specification |
| --- | --- |
| Certificate of Analysis (COA) | Required for every delivery. |
| Safety Data Sheet (SDS) | Latest revision to accompany the first delivery and upon revision thereafter. |
| Delivery Note | Required for every delivery indicating product, quantity and batch number. |
| Weighbridge Ticket | Required for all bulk deliveries. |
| Batch Identification | Product batch number to be clearly stated on all packaging and delivery documentation. |
| Manufacturing Date | To be clearly indicated on each container or bag. |
| Expiry Date | Where applicable, to be clearly indicated on packaging. |
| Product Labelling | All packaging shall comply with the Globally Harmonized System (GHS) and applicable South African regulations for hazardous chemicals. |

| Requirement | Minimum Requirement |
| --- | --- |
| Product Quality | Chemicals shall be manufactured under a documented Quality Management System, preferably certified to ISO 9001 or an equivalent internationally recognised quality |

|  | management standard. |
| --- | --- |
| Contaminants and Impurities | Chemicals shall be free from impurities, heavy metals or other contaminants that could adversely affect the water treatment process, damage plant equipment, contaminate the treated water, or compromise compliance with the applicable water use licence and environmental discharge requirements. Suppliers shall declare any known impurities and provide supporting analytical data upon request. |
| Product Certification | Preference will be given to chemicals certified to applicable South African Bureau of Standards (SABS) specifications and/or NSF/ANSI standards for water treatment chemicals (e.g. NSF/ANSI/CAN 60), where such certification is available for the product. Equivalent internationally recognised certifications may also be considered. |